REGULATIONS FOR TRAIN SIGNALLING ON SINGLE LINES BY THE ELECTRIC TOKEN BLOCK SYSTEM.

(Except where otherwise provided the term Electric Token includes Electric Staff or Tablet)

# BELL SIGNALS

See						         Beats

Reg.					    	          on Bell.	How to be given.

1.	*Call attention					1	1

1.	*EMERGENCY “CALL ATTENTION” SIGNAL (TO 		A NUMBER OF

PRECED EMERGENCY BELL SIGNALS ONLY)		BEATS IN RAPID

SUCCESSION

IS LINE CLEAR FOR: -

Express passenger train, newspaper train or break-

down van train or snow plough going to clear the line

or light engine going to assist disabled train (“A”

Headcode)? …………………………………………….	4	Consecutively.

Officers’ Special not requiring to stop in section (“A”

Headcode)? …………………………………………….	4	Consecutively.

Ordinary passenger train, mixed train or breakdown

van train NOT going to clear the line or loaded rail

motor train (“B” Headcode)? ………………………….	4	3 pause 1.

Branch Passenger train (“B” Headcode)? ……………	4	1 pause 3.

EXPRESS DIESEL CAR (“A” Headcode)? …………..	8	4 pause 1 pause 3	ORDINARY PASSENGER OR PARCELS DIESEL

CAR (“B” Headcode)? …………………………………..	9	5 pause 1 pause 3	Parcels, fish, fruit, horse, livestock, meat, milk, pigeon

or perishable train composed entirely of vehicles

conforming to coaching stock requirements (“C”

Headcode)? …………………………………………….	5	1 pause 3 pause 1	Express freight, livestock, perishable or ballast  train

pipe fitted throughout with the automatic brake

3&4	  operative on not less than half of the vehicles (“C”

Headcode)?  ……………………………………………	5	3 pause 1 pause 1	Empty coaching stock train (not specially authorised to

carry “A” Headcode), or empty rail motor train

(“C” Headcode)?	  ………………………………………	5	2 pause 2 pause 1	Express freight, livestock, perishable or ballast train

partly fitted with the automatic brake operative on

not less than one third of the vehicles (“D” Head-

code)?  ………………………………………………….	5	Consecutively.

Express freight, livestock, perishable or ballast train

partly fitted with NOT less than four braked vehicles

connected by vacuum pipe to the engine (“E” Head-

code)?	………………………………………………….	5	1 pause 2 pause 2

Express freight, livestock, perishable or ballast train

with a limited load of vehicles NOT fitted with con-

tinuous brake (“E” Headcode)?  ……………………...	5	1 pause 2 pause 2

Express freight, livestock, perishable or ballast train

NOT fitted with continuous brake (“F” Headcode)?…	5	3 pause 2

Light engine or light engines coupled (“G” Headcode)	5	2 pause 3

Engine with not more than two brake vans (“G”

Headcode)?  …………………………………………….	5	1 pause 1 pause 3

## BELL SIGNALS

See 					              		     Beats		How to be given

Reg.							     on Bell

Through freight or ballast train not running under class

“C,” “D,” “E” or “F” Headcode (“H” Headcode)?  ……	5	1 pause 4

Mineral or empty wagon train (“J” Headcode)?  ……..	5	4 pause 1

Freight, mineral or ballast train stopping at inter-

3&4	  mediate stations (“K” Headcode)?	…………………..	3	Consecutively

Branch freight train (“K” Headcode)?  ………………...	3	1 pause 2

TRAIN CONVEYING OUT OF GAUGE OR EX-

CEPTIONAL LOAD?  ………………………………….	10	2 pause 6 pause 2

3&5& 8	Freight, ballast or Officers’ Special train, requiring to

stop in section (“K” Headcode)?  …………………….	7	2 pause 2 pause 3

3&5&9	Trolley requiring to go into or pass through tunnel? …	5 	2 pause 1 pause 2

3	*Train entering section  …………………………………	2	Consecutively

5	*Section clear but station or junction blocked	 ……….	13	3 pause 5 pause 5

6	*Engine assisting in rear of train  ………………………	4	2 pause 2

10&12	*Train out of section or obstruction removed	………..	3	2 pause 1

12	*Obstruction Danger  ……………………………………	6	Consecutively

13	Release Token  ………………………………………….	7	5 pause 2

13	Token replaced  …………………………………………	7	2 pause 5

16	Blocking Back  …………………………………………..	6	3 pause 3

17	*Stop and examine train  ………………………………	7	Consecutively

18	Cancelling “Is line clear?” or “Train entering section”

signal  …………………………………………………..	8	3 pause 5

9 consecutively TO							TOKEN STATION

IN ADVANCE

19		*Train passed without tail lamp  ………………………	9	4 pause 5 TO

TOKEN STATION

IN REAR

20	*Train divided  …………………………………………..	10	5 pause 5

21	Shunt train for following train to pass…………………	11	1 pause 5 pause

22	*Train or vehicles running away  ……………………..	12	2 pause 5 pause

24	Opening of Token Station	……………………………	15	5 pause 5 pause

27	Testing Instruments, bells and gongs  ………………	16	Consecutively

28	Time  ……………………………………………………	18	8 pause 5 pause

29	Lampman or Fog-signalman required  ……………...	19	9 pause 5 pause

30	Testing controlled or slotted signals  ………………..	20	5 pause 5 pause 5 							pause 5

37	TAKE SLOT OFF -  TRAIN WAITING		7	3 pause 4

THE ORDINARY “CALL ATTENTION” SIGNAL (1 BEAT) MUST PRECEDE ALL SIGNALS

EXCEPT THOSE MARKED THUS * (SEE REGULATION 1).